A GUIDE FOR CONDUCTING FACILITY PRE-FIRE PLANS

- Sketch Each Floor

- Show Roof access of venting devices

- Show approaches to the facility

- Indicate the size of the facility (Length and Width)

- Show location of attic access doors, hatches, or ladders

- Show roof access ladders

- Show the location of the master electrical shutoff(s), other electrical shutoffs should be shown.

- Show the location of the master gas/LPG shutoff.

- Show the Direction of north by an arrow.

- Show specific types of doors for other than standard personnel doors.

- Show Fences and gates.    NOTE: Symbols are used to standardize and simplify information on the sketches the symbols that will be used are on included in this packet.

- Show all major partitions and doors.

- Show room numbers if they are marked on the facility, otherwise indicate the use of the room.

- Show the location of stairs and stairwells.

- Show the locations of elevators.

- Show locations of firewalls, and fire doors.

- Show fire or smoke partitions in concealed spaces such as attics.

- Show locations of fire hydrants and other available water sources.

- Show the location of fire alarm control panels

- Show the location of annunciator panels.

- Show the location of all carbon dioxide, FM200, or clean agent fire extinguishing systems.

- Show the location of all sprinkler risers, if applicable.

- If the facility is only partially sprinkled, indicate areas that are protected.

- If the facility has more than one sprinkler riser, indicate the area protected by each riser.

- If the facility does not have a sprinkler system or if the facility is protected completely by a sprinkler system place the proper symbol at the top of the sketch.

- Show the location of all Post Indicator Valves and show which risers the valves control.

- Show the location of all standpipes, hose cabinets and outlets.

- Show the location of any installed fire pumps.

- Indicate the location of hazards in the building, such as explosives, radiation, flammable liquids, toxics, ect. That constitute an uncommon hazard to the firefighter.

- Indicate the location of where Material Safety Data Sheets (MSDS) are stored.

West Side RFPD

Station #2

1185 Tucker Road

Hood River, Oregon 97031

Dear Business owner,

Enclosed you will find a graph sheet in which we are requesting you draw the outline of your business.   This will help us in case of an emergency should one arise.  Please show the location of the following items in your drawing,

- Doors and windows

- Power Shutoff

- Gas/LPG Shutoff

- Sprinkler Connection

- Fire Alarm Panel

- Non-Usable Doors or Windows

- Any Special Hazards (Explosives, Chemicals, Etc.)

At the bottom of this drawing please list the following,

- Type of Business you conduct

- Emergency Contact phone numbers

- Number of employees normally on site.

- Business Name and Address.

- Insurance Company

Please return your drawings to the following address or contact us at the following numbers listed below:

1185 Tucker Road, Hood River, OR 97031

Fire and Life Safety Preplans

Attn: Administration

541-386-1550

josh.beckner@westsidefire.com

www.westsidefire.com

Pre-Fire Plan Data Entry Form

Date: _________________________		Fire Department/District: __West Side RFPD___________________

Business Name: _____________________________________________________________________________________

Address: _________________________________________________________    Bldg/Suite #: __________________

City: _____________________________________________________________	  Phone #:______________________

Type of Business: __________________________________________________ Dept. Map Page #: _______________

Owner Name: _____________________________________________________	Phone #:______________________

Manager Name: __________________________________________________	Phone #:______________________

Other Name: _____________________________________________________	Phone #:______________________

Nearest Cross-Street is: _______________________________________________________________________________

Primary Hydrant Number: _______________	Located at: _________________________________________________

Other water source is: _______________________________   Square footage: ________ Stories in Height: __________

Occupancy Class Code: __________	 Location of Heating: ________________________________________________

Construction Class Code: ________   Gas Shutoff: ____________________________    Type of Heat: Natural Gas  LPG

Electrical Shutoff Location:  ____________________________________ Alarm Panel: ____________________________

Basement: YES NO   Remarks: _____________________________________________________________________

Elevator: YES NO   Remarks: _____________________________________________________________________

Lock Box: YES NO   Remarks: _____________________________________________________________________

F.D.C.: YES NO   Remarks: ________________________________________________________________________

Standpipes: YES NO   Remarks: ____________________________________________________________________

Sprinkler System: YES NO   Remarks: _______________________________________________________________

# of Sprinkler Risers: ___________   Remarks: _____________________________________________________________

Location of Sprinkler Controls: ___________________________________________________________________________________

Location of Water Shutoff Valves: ________________________________________________________________________________

Exposure Information: _________________________________________________________________________________________

COMMENTS: _________________________________________________________________________________________________

| FIRE DEPARTMENT |  |  |  |  | West Side RFPD |  |  |  | PRE-PLAN # |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Business Name: |  |  |  |  |  |  |  |  |  | District |  |  |  |
| Emergency Contacts: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Occupancy and Hazards: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Construction: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire Protection Systems: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water Supplies: |  |  | Nearest Large Volume Water Supply: |  |  |  |  |  |  |  |  |  |  |
| Water Supply-Needed Fire Flow: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INITIAL DISPATCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Involvement | GPM |  |  | ENGINES |  | LADDERS | CHIEFS | SPECIAL UNITS |  |  | EMS |  | OTHERS |
| 50 % |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100% |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPECIAL RESCOURCES: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EXPOSURES: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STRATEGIES: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |

PRE-INCIDENT PLAN FORM

SAMPLE COMPLETED PRE-PLAN FORM

| FIRE DEPARTMENT |  |  |  |  | Southwest Fire Department |  |  |  |  |  | PRE-PLAN # |  | 2011-14 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address: | 147 South 7th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Business Name: |  | Frank’s Shooting Sports |  |  |  |  |  |  |  | District SWFD |  |  |  |  |
| Emergency Contacts: |  |  |  | Frank Martinez, Owner (505) 555-1234 Jim Romero, Manager (505) 555-5678 ABC Alarm Co. (505) 555-9112 |  |  |  |  |  |  |  |  |  |  |
| Occupancy and Hazards: |  |  |  | Gun Shop, with Bulk Ammunition storage, and shooting supplies. Ammunition is stored In Back room in concrete enclosure, solvents in fire proof lockers. Will have to force entry into store and ensure all emergency escape routes are clear before making an interior attack. |  |  |  |  |  |  |  |  |  |  |
| Construction: |  |  |  | Steel Frame Building. |  |  |  |  |  |  |  |  |  |  |
| Fire Protection Systems: |  |  |  | Full Sprinkler System, with Clean agent extinguishing system in Weapons/Ammo Vault. |  |  |  |  |  |  |  |  |  |  |
| Water Supplies: |  |  |  | Hydrant located at 7th street and 147th St. Hydrant provides 1,000 GPM @ 75 PSI. Dispatch will send 2 County Fire Districts with tankers for support. If tankers are not needed, personnel sent will be used on fire ground. Nearest Large Volume Water Supply: As Above or Hydrant at 147th and Pine 200 Ft. South of above hydrant |  |  |  |  |  |  |  |  |  |  |
| Water Supply- Needed Fire Flow: |  |  |  | INITIAL DISPTACH |  |  |  |  |  |  |  |  |  |  |
| Involvement |  | GPM |  |  |  | ENGINES | LADDERS | CHIEFS | SPECIAL UNITS |  |  | EMS |  | OTHERS |
| 50 % |  | 1500 GPM |  |  |  | E-1 & E-3 |  | 1 | R-1 & Police Dept |  |  |  |  | County Tankers, As needed |
| 100% |  | 2800 GMP |  |  |  | E-1, E-3 & E-7 |  | 1 | R-1, 7 Police Dept |  |  |  |  | County Tankers, As needed |
| SPECIAL RESCOURCES: |  |  | Tankers from County Fire Districts can be sent to the City Yards, at 155 7th street if a fill site for drafting is needed. |  |  |  |  |  |  |  |  |  |  |  |
| EXPOSURES: |  |  | Side-B: Exposure to Sam’s Cabinet Shop, wood frame construction Side-D Exposure to Pete’s Pet Shop metal frame 10’ clearance between buildings. |  |  |  |  |  |  |  |  |  |  |  |
| STRATEGIES: |  |  | Disconnects for Gas and Electric on Side D of structure. Protect exposure on Sides B & D. |  |  |  |  |  |  |  |  |  |  |  |
| COMMENTS: |  |  | Police Department must be requested to help in securing the scene as this is a fire arms store, which stores ammunition and weapons. They will be needed to take custody of the scene upon completion of any fire suppression. |  |  |  |  |  |  |  |  |  |  |  |

PRE-INCIDENT PLAN DIAGRAM

| Direction N | Address: 147 South 7th Street |  | PRE-PLAN # 2011-14 |
| --- | --- | --- | --- |
|  | Name of Occupancy: Frank’s Shooting Sports | DISTRICT 1 |  |
| Oak Street B Main Show Room Fully Sprinkled Displays are combustible 1100 Sqft A 147St C Weapons & Ammo Vault 650 sqft Clean Agent Suppression Stock Room Heavy Combustibles, and flammable cleaners FDC SPRK RISER D Electrical Disconnect Hydrant 1,000 GPM Gas Meter 7th Street |  |  |  |

NOT TO SCALE

PRE-INCIDENT PLAN DIAGRAM

| Direction | Address: | PRE-PLAN # |
| --- | --- | --- |
|  | Name of Occupancy: |  |

NOT TO SCALE